### System Architecture Overview

This is a client-server model optimized for edge computing:

- Client (Edge Node): A Raspberry Pi (or similar) handles the hardware interfacing (motion sensing, camera capture) and the heavy processing (OCR) locally. This minimizes network bandwidth.

- Server (Backend): A central server (Node.js/Python) manages the database of registered vehicles and exposes an API endpoint for the client.

- Database: Stores authorized license plate numbers and corresponding owner information.

### Phase 1: The Client (Edge Device)

The client is typically a Raspberry Pi 4 or 5 equipped with a camera module.

#### 1. Hardware Required

- Raspberry Pi (4GB RAM recommended for smooth OCR)

- Raspberry Pi Camera Module v2/v3

- PIR (Passive Infrared) Motion Sensor

- Jumper Wires and Breadboard

- Relay VCC -> Raspberry Pi 5V (Pin 2 or 4)

- Relay GND -> Raspberry Pi GND (Pin 6 or similar)

- Relay IN (Signal) -> Raspberry Pi GPIO 23 (Pin 16)

- Relay COM & NO (Normally Open) -> Connected to the gate controller's wall-switch or trigger terminals.

#### 2. Sensing Motion (Wiring)

Connect the PIR sensor:

- VCC -> Pi 5V (Pin 2 or 4)

- GND -> Pi GND (Pin 6 or similar)

- OUT (Data) -> GPIO Pin (e.g., GPIO 17, Pin 11)

#### 3. Client Logic (Python)

The client script must run continuously, waiting for a trigger, processing the image, and sending the results.

Key Libraries:

- picamera (or opencv-python)

- Pillow (PIL) for image manipulation

- pytesseract (Wrapper for Tesseract OCR Engine)

- requests (for sending API calls)

### Phase 2: The Server

The server receives the data, queries the database, and decides whether access should be granted.

#### 1. Technologies

- Language: JavaScript (Node.js) or Python (Flask/FastAPI)

- Database: SQL (PostgreSQL/MySQL) or NoSQL (MongoDB). We will use PostgreSQL (SQL) for its robust string matching and structural integrity.

2. Database Schema (PostgreSQL)

3. Server Logic (Node.js with Express and pg)

### Summary of Data Flow

- A car pulls up. The PIR Sensor detects motion.

- The Client (Raspberry Pi) triggers the Camera Module to take a snapshot (arrival.jpg).

- The Client uses Pytesseract (OCR) to convert the image of the license plate into the string "CALIF123".

- The Client sends an HTTP POST request containing {"license_plate": "CALIF123"} to the Server's IP address.

- The Server receives the request and executes a SQL query on PostgreSQL: SELECT ... WHERE license_plate = 'CALIF123'.

- The Server finds a match and responds to the Client with {"registered": true, "owner_name": "John Doe"}.

- The Client receives this JSON response and acts accordingly (e.g., outputs "ACCESS GRANTED" or activates a relay to open a gate).